Sextractor Caveats

- Background detection

  - Sextractor incorrectly registers the background level as NAN and finds no sources for images with NAN values.  Two workarounds for this problem that we found to be effective are:

    - Edit image so that NANs are changed to a value less than or equal to –BIG.  BIG is defined in the sextractor header files as 1e+30, if a pixel is smaller than this then sextractor will ignore it.

    - Perform manual estimation of background by editing default.sex configuration file parameters as follows: “BACK_TYPE MANUAL”, “BACK_VALUE  <background value>”

  - If the background contains no noise then Sextractor will assume that the image contains constant values and will not extract anything (this is relevant for our synthetic pattern files). A workaround for this that we found to be effective is to set “BACK_TYPE MANUAL” and “BACK_VALUE 0.0” in default.sex configuration file.  This effectively turns off automatic background detection.

- Sextractor .param file notes:

  - FLUX_ISO – this is the value that should be used to detect flux, otherwise Sextractor will detect flux in pattern files incorrectly.

  - Sextractor can also be configured to estimate errors on all parameters, but the error estimates seem inaccurate and therefore for our purposes they are ignored.

  - Sextractor can be configured to give source locations in several types of coordinates:

    - X_IMAGE, Y_IMAGE – we found these to work correctly in our tests.

    - Many other coordinate types were found to not work as documented.  For example, ALPHA_J2000, DELTA_J2000 resulted in native coordinates without the expected conversion to J2000.

  - In the default.sex configuration file the following should be set when running Sextractor on our synthetic pattern files used for testing:  “DETECT_MINAREA 1”.  Not doing this results in failure to extract some of the sources in our synthetic pattern files, which have sources that are contained in a single pixel.

  - Sextractor seems to assume that pixel values are actual fluxes and not flux densities.  Therefore it is necessary to create a temporary image in which the fluxes are converted to flux densities.

  - We were unable to get Sextractor to accurately extract sources from images with a lot of NANs, even when the NAN workarounds described above are applied.

  - The image size affected the sensitivity of extraction in our tests, i.e., extraction parameters had to be changed significantly for images of different sizes.